Machine Learning Safety - Exercise-2

2.1}

·  Loss: An unwanted outcome that causes harm or damage.
Example: A self-driving car hits a pedestrian, causing injury.

·  Hazard: A system condition that could potentially lead to a loss.
Example: The vehicle fails to detect a pedestrian crossing the road.

·  Risk: The likelihood and severity of a hazard leading to a loss.
Example: Driving at high speed with unreliable pedestrian detection increases the risk of an accident.

2.2}

| Dimension | Operating Conditions | Non-Operating Conditions | Runtime Detection of ODD Violation |
| --- | --- | --- | --- |
| Weather | Clear and sunny weather | Heavy rain, fog, snow, storms | Weather sensors or low camera visibility detection |
| Lighting | Daytime with good natural lighting | Nighttime, tunnels, low-light conditions, glare | Brightness analysis from camera images |
| Camera Condition | Clean and unobstructed front camera | Dirty lens, water drops, blurred or blocked camera | Image quality checks for blur, occlusion, or low contrast |
| Scene Type | Normal urban/suburban roads with standard traffic | Off-road areas, construction zones, highly unusual environments | Scene classification or map comparison |
| Vehicle Speed | Moderate city-driving speeds | Very high-speed driving where reaction time is limited | Vehicle speed monitoring from onboard sensors |

The ODD for this CARLA perception system is limited to daytime, clear-weather urban driving with a clean camera and moderate driving speeds. Any condition outside these assumptions should be treated as an ODD violation because the model was not designed or trained for those environments.

2.3}

| ID | Loss Description | Why Unacceptable |
| --- | --- | --- |
| L-1 | A pedestrian is injured or killed by the vehicle. | Human injury or loss of life is the most critical safety failure. |
| L-2 | The vehicle collides with another vehicle. | Collisions can cause injuries, fatalities, and major property damage. |
| L-3 | The vehicle ignores or misinterprets a traffic light. | This can lead to dangerous traffic violations and accidents. |
| L-4 | The vehicle behaves unpredictably due to incorrect model predictions. | Unstable driving behavior reduces safety and trust in the system. |
| L-5 | Damage is caused to public or private property. | Property damage can create financial loss and unsafe road conditions. |
| L-6 | The perception system fails in conditions outside its ODD without warning. | Operating outside safe conditions increases the chance of accidents. |

2.4}

| ID | Hazard | Loss(es) | Likelihood | Severity |
| --- | --- | --- | --- | --- |
| H-1 | The system fails to detect a pedestrian in front of the vehicle. | L-1, L-4 | Medium | High |
| H-2 | The system fails to detect another vehicle nearby. | L-2, L-4 | Medium | High |
| H-3 | The system incorrectly detects or misses a traffic light signal. | L-2, L-3 | Medium | High |
| H-4 | The perception system operates outside its defined ODD without detecting it. | L-1, L-2, L-6 | High | High |
| H-5 | False positive detections cause unnecessary braking or unsafe maneuvers. | L-2, L-4, L-5 | Medium | Medium |
| H-6 | Poor camera input quality (blur, occlusion, glare) leads to unreliable predictions. | L-1, L-2, L-3 | Medium | High |

| ID | Controller | Control Action | UCA Type | Hazard(s) | Unsafe Scenario |
| --- | --- | --- | --- | --- | --- |
| UCA-1 | Planning Module | Brake command | Not provided | H-1, H-2 | The planner does not issue a brake command when a pedestrian or vehicle is detected ahead. |
| UCA-2 | Planning Module | Brake command | Provided unsafely | H-5 | The planner suddenly brakes even though no obstacle present causing unsafe driving behavior. |
| UCA-3 | Planning Module | Steering command | Wrong timing / order | H-2, H-4 | The vehicle changes direction too late to avoid a nearby vehicle. |
| UCA-4 | Planning Module | Acceleration command | Wrong duration | H-3, H-4 | The vehicle continues accelerating for too long near a traffic light or obstacle. |
| UCA-5 | Human Operator | Override command | Not provided | H-1, H-4 | The human operator fails to take control when the perception system behaves incorrectly. |
| UCA-6 | Human Operator | Override command | Provided unsafely | H-2, H-5 | The operator unnecessarily overrides the system and causes unsafe vehicle movement. |
| UCA-7 | Human Operator | Steering intervention | Wrong timing / order | H-1, H-2 | The operator reacts too late to avoid a collision. |
| UCA-8 | Human Operator | Manual braking | Wrong duration | H-5 | The operator brakes too briefly, causing the vehicle to continue moving toward a hazard. |

2.6}

2.7}

| UCA | Safety Constraint | Level | Verification |
| --- | --- | --- | --- |
| UCA-1 | The system must issue a brake command whenever a pedestrian or vehicle is detected within a dangerous distance. | System-level | Test emergency braking behavior in simulation scenarios. |
| UCA-2 | The planner must confirm obstacle detection confidence before triggering automatic braking. | Model-level | Evaluate false positive rate of perception models. |
| UCA-3 | Steering commands must be generated within a safe response time after obstacle detection. | System-level | Measure controller response latency during testing. |
| UCA-4 | The vehicle must reduce or stop acceleration when approaching detected traffic-related hazards. | System-level | Test vehicle behavior near traffic lights and obstacles. |
| UCA-5 | The human operator must receive clear alerts when the system detects uncertain or unsafe conditions. | System-level | Verify warning and takeover mechanisms in simulations. |
| UCA-6 | Manual override actions must require confirmation or safe operating conditions before execution. | System-level | Test operator override procedures and safety checks. |
| UCA-7 | The system must provide timely hazard notifications to support fast operator reactions. | System-level | Evaluate alert timing and operator reaction performance. |
| UCA-8 | Braking assistance must remain active until the vehicle reaches a safe state. | System-level | Test stopping distance and brake duration behavior. |

2.8}

| UCA | Causal Scenario | Root Cause | Related Constraint |
| --- | --- | --- | --- |
| UCA-1 | The planner does not send a brake command because the pedestrian detector fails to recognize a pedestrian at night. | Incorrect feedback from the perception model caused by training only on daytime data. | The system must issue a brake command whenever a pedestrian or vehicle is detected within a dangerous distance. |
| UCA-2 | The vehicle brakes suddenly even though no obstacle exists because the model falsely detects a pedestrian. | False positive prediction due to noisy or unclear camera input. | The planner must confirm obstacle detection confidence before triggering automatic braking. |
| UCA-3 | The steering command is issued too late to avoid a nearby vehicle. | Timing delay in perception processing or planning response. | Steering commands must be generated within a safe response time after obstacle detection. |
| UCA-4 | The vehicle continues accelerating toward an intersection because the planner assumes traffic-light presence means it is safe to continue. | Flawed internal model and missing traffic-light state information. | The vehicle must reduce or stop acceleration when approaching detected traffic-related hazards. |
| UCA-5 | The human operator does not take control when the perception system behaves incorrectly. | Human attention lapse or unclear system warning. | The human operator must receive clear alerts when the system detects uncertain or unsafe conditions. |
| UCA-6 | The operator unnecessarily overrides the autopilot and causes unsafe steering behavior. | Mode confusion or misunderstanding of vehicle status. | Manual override actions must require confirmation or safe operating conditions before execution. |
| UCA-7 | The operator reacts too late because the hazard warning appears only moments before collision. | Delayed feedback from the system. | The system must provide timely hazard notifications to support fast operator reactions. |
| UCA-8 | The operator releases the brake too early and the vehicle continues moving toward the obstacle. | Human error or incorrect judgment of stopping distance. | Braking assistance must remain active until the vehicle reaches a safe state. |